| Shenzhen ProMagTech Co., Ltd. 3F, Bldg 1, No.22 Dongda Rd., Dongkeng Community, Fenghuang St., Guangming Dist., Shenzhen, China | OUTGOING INSPECTION REPORT OQC - Multi-Product Summary | Report No.: OQC-20260521-001 Date: 2026-05-15 |
| --- | --- | --- |

____________________________________________________________________________________________________

| Customer: |  |  | Lot No.: |  |  | Qty Inspected: |  |  | Inspector: |  |  | Conclusion: |  |  | PASS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | TEST ITEM | TERMINAL |  | SPECIFICATION | YCPQ2618-350µH |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Value |  | Result | Value | Result | Value | Result |  | Value | Result |  | Value | Result |  |
| 1 | Inductance (L) | L(4-6) |  | 350µH ±7% | 347.2µH |  | PASS | 148.5µH | PASS | 99.3µH | PASS |  | 67.1µH | PASS |  | 218.6µH | PASS |  |
| 2 | DC Bias Inductance (Ldc) | L(4-6) |  | ≥L0A×80%(25°C) | 85.5% |  | PASS | 87.2% | PASS | 89.1% | PASS |  | 90.3% | PASS |  | 86.8% | PASS |  |
| 3 | Leakage Inductance | 4-6 |  | 7µH MAX | 4.8µH |  | PASS | 2.9µH | PASS | 2.1µH | PASS |  | 1.7µH | PASS |  | 3.5µH | PASS |  |
| 4 | Turns Ratio | (N1+N4):N2:N3 |  | 30:9:6 | 30.0:9.0:6.0 |  | PASS | 20.0:6.0:4.0 | PASS | 16.0:5.0:3.0 | PASS |  | 12.0:4.0:3.0 | PASS |  | 24.0:7.0:5.0 | PASS |  |
| 5 | DCR (N1) | 4-6 |  | 55 mΩ max | 42.3mΩ |  | PASS | 28.1mΩ | PASS | 21.6mΩ | PASS |  | 17.2mΩ | PASS |  | 33.5mΩ | PASS |  |
| 6 | DCR (N2) | 8,9-10,11 |  | 25 mΩ max | 18.7mΩ |  | PASS | 11.5mΩ | PASS | 9.2mΩ | PASS |  | 7.8mΩ | PASS |  | 14.8mΩ | PASS |  |
| 7 | DCR (N3) | 1-2 |  | 18 mΩ max | 13.2mΩ |  | PASS | 8.6mΩ | PASS | 7.1mΩ | PASS |  | 5.4mΩ | PASS |  | 10.1mΩ | PASS |  |
| 8 | Inter-turn Test | 4-6 |  | / | OK |  | PASS | OK | PASS | OK | PASS |  | OK | PASS |  | OK | PASS |  |
| 9 | Hi-Pot (PRI–SEC) | PRI–SEC |  | AC 3000V / 2mA / 60s | OK |  | PASS | OK | PASS | OK | PASS |  | OK | PASS |  | OK | PASS |  |
| 10 | Hi-Pot (PRI–CORE) | PRI–CORE |  | AC 3000V / 2mA / 60s | OK |  | PASS | OK | PASS | OK | PASS |  | OK | PASS |  | OK | PASS |  |
| 11 | Hi-Pot (SEC–CORE) | SEC–CORE |  | AC 1000V / 2mA / 60s | OK |  | PASS | OK | PASS | OK | PASS |  | OK | PASS |  | OK | PASS |  |
| 12 | Insulation Resistance | CORE-COIL / COIL-COIL |  | 100MΩ MIN / DC 500V | >500MΩ |  | PASS | >500MΩ | PASS | >500MΩ | PASS |  | >500MΩ | PASS |  | >500MΩ | PASS |  |

REMARKS : All test items for the above 5 products are within specification. Products are approved for shipment.

| INSPECTION CONCLUSION : ✓ ACCEPT □ REJECT |  |  |
| --- | --- | --- |
| Inspected by: ______________________ | Checked by: ________________________ | Approved by: _______________________ |

Shenzhen ProMagTech Co., Ltd.  |  OQC Report  |  Page 1 of 1